CUDA Programming – Lab 6 Report

Parallel Computing on NVIDIA GeForce RTX 2050

GPU: NVIDIA GeForce RTX 2050  |  Compute Capability: 8.6 (Ampere)  |  CUDA 13.1

# Program 1: GPU Device Properties Query

This program queries and displays the hardware specifications of all available CUDA-capable devices using the cudaDeviceProp structure. Understanding these properties is essential for tuning kernel launch parameters.

## Device Specifications – NVIDIA GeForce RTX 2050

| Property | Value |
| --- | --- |
| GPU Name | NVIDIA GeForce RTX 2050 |
| Compute Capability | 8.6 (Ampere Architecture) |
| Global Memory | 4,095 MB (~4 GB GDDR6) |
| Shared Memory / Block | 48 KB |
| Constant Memory | 64 KB |
| Max Threads / Block | 1,024 |
| Max Block Dimensions | (1024, 1024, 64) |
| Max Grid Dimensions | (2,147,483,647 × 65,535 × 65,535) |
| Warp Size | 32 threads |
| CUDA Toolkit | 13.1 (compiled with -allow-unsupported-compiler -gencode arch=compute_86,code=sm_86) |

## Key Observations

The maximum of 1,024 threads per block with a warp size of 32 means there are at most 32 warps per block.

The 48 KB shared memory per block is sufficient for tile-based optimizations (e.g., tiled matrix multiplication).

The massive grid dimension limit (2.1 billion in X) allows processing arbitrarily large datasets in 1D kernels.

Compute Capability 8.6 supports asynchronous memory copies, Tensor Cores, and structured sparsity operations.

The toolkit/driver version mismatch (CUDA 13.1 toolkit vs driver supporting 13.0) required explicit -gencode sm_86 to avoid runtime PTX rejection — an important practical lesson in CUDA deployment.

# Program 2: Parallel Sum Reduction

Parallel reduction is a fundamental GPU pattern that sums N values using a logarithmic tree of additions in shared memory. Each block reduces its tile to a single partial sum, and the GPU results are accumulated on the CPU. The problem size is fixed at N = 1,048,576 (2²⁰) float elements (~4 MB).

## Kernel Design

Each thread loads one element into shared memory (sdata[tid]).

Threads synchronize (__syncthreads()), then reduce in log₂(blockDim.x) steps.

Thread 0 of each block writes the block's partial sum to d_out[blockIdx.x].

Final summation of partial sums is done on the CPU.

## Strong Scaling Results  (CPU baseline: 6.26 ms)

| Threads/Block | Blocks/Grid | GPU Time (ms) | Speedup vs CPU | Efficiency |
| --- | --- | --- | --- | --- |
| 32 | 32,768 | 0.2508 | 24.6× | 100% (baseline) |
| 64 | 16,384 | 0.1718 | 35.9× | 73% |
| 128 | 8,192 | 0.1606 | 38.4× ✓ BEST | 39% |
| 256 | 4,096 | 0.1787 | 34.5× | 17.5% |
| 512 | 2,048 | 0.1810 | 34.1× | 8.7% |
| 1024 | 1,024 | 0.2449 | 25.2× | 3.2% |

Efficiency = (relative GPU speedup / thread-count ratio) × 100. It measures how well additional threads translate into proportional performance gains.

Figure 1: Speedup vs Threads/Block – Parallel Reduction (1M floats, fixed N)

## Conclusions – Parallel Reduction

BEST CONFIG: 128 threads/block → 38.4× speedup over CPU (0.16 ms vs 6.26 ms).

Performance peaks at 128 threads/block because it best balances warp occupancy, shared memory usage, and reduction tree depth on the RTX 2050 (SM count × warp schedulers).

Efficiency drops sharply with more threads — classic Amdahl's Law: the serial portion (final CPU summation of partial block results) limits maximum theoretical speedup.

At 1024 threads/block, fewer blocks are launched, reducing SM utilization and hiding less memory latency — performance degrades back toward 32-thread levels.

For 1M elements, the kernel is compute-bound within shared memory, making it well-suited for GPU.

Practical lesson: a warm-up run is essential; first-launch JIT overhead can skew measurements by 10–20×.

# Program 3: Integer Matrix Addition

Two large integer matrices A and B of size 4096×4096 are added element-wise to produce C = A + B. The kernel uses a 2D thread/block layout: each thread computes exactly one element of the output matrix. Problem size is fixed at 4096×4096 = 16,777,216 integers (~192 MB total for A, B, C).

## Kernel Analysis

| Metric | Value | Formula |
| --- | --- | --- |
| Floating-point Ops (FLOPs) | 16,777,216 | 1 ADD per element → N² |
| Global Memory Reads | 33,554,432 | 2 reads per element (A[i] and B[i]) → 2×N² |
| Global Memory Writes | 16,777,216 | 1 write per element (C[i]) → N² |
| Arithmetic Intensity | 0.0833 FLOPs/byte | 1 FLOP ÷ 12 bytes (2 reads + 1 write × 4 bytes) |
| Memory Classification | MEMORY-BOUND | AI << 1 → bottleneck is bandwidth, not compute |

Because each element requires only 1 addition but 3 global memory transactions (2 reads + 1 write), this kernel is extremely memory-bandwidth limited. The RTX 2050 peak memory bandwidth is ~192 GB/s; at peak, 192 MB would transfer in ~1 ms — matching the observed ~2.5 ms (accounting for latency and overhead).

## Strong Scaling Results  (CPU baseline: 203.37 ms)

| Block Shape | Threads/Block | Blocks/Grid | GPU Time (ms) | Speedup vs CPU | Throughput (GFLOP/s) | Correct? |
| --- | --- | --- | --- | --- | --- | --- |
| 4×4 | 16 | 1024×1024 | 3.4863 | 58.3× | 4.81 | ✓ YES |
| 8×8 | 64 | 512×512 | 2.5341 | 80.3× | 6.62 | ✓ YES |
| 16×16 | 256 | 256×256 | 2.5171 | 80.8× ✓ BEST | 6.67 | ✓ YES |
| 32×32 | 1024 | 128×128 | 2.5703 | 79.1× | 6.53 | ✓ YES |

Figure 2: Speedup vs Block Shape – Matrix Addition (4096×4096)

Figure 3: GPU Compute Throughput (GFLOP/s) vs Block Shape – Matrix Addition

## Conclusions – Matrix Addition

BEST CONFIG: 16×16 block → 80.8× speedup over CPU (2.52 ms vs 203.37 ms).

This is a textbook memory-bandwidth-bound kernel: arithmetic intensity = 0.083 FLOPs/byte. GPU gains come entirely from higher memory bandwidth, not compute.

The 4×4 block (16 threads) is noticeably slower: fewer threads per block means less warp-level parallelism to hide global memory latency, leading to 58× vs 80× speedup.

Scaling plateaus after 64 threads/block — the GPU memory subsystem is already saturated. This confirms Amdahl's Roofline: we have hit the memory bandwidth ceiling, not compute.

32×32 blocks (1024 threads) produce slightly fewer blocks (128×128 = 16,384 blocks), marginally reducing SM utilization and causing a slight performance dip vs 16×16.

All configurations produce numerically correct results, validating kernel correctness across all block sizes.

The CPU takes 203 ms for the same operation — the GPU is ~80× faster, demonstrating how even simple, memory-bound kernels benefit massively from GPU parallelism.

# Overall Summary & Comparison

| Program | Pattern | Best Config | CPU Time | GPU Time | Peak Speedup | Bottleneck |
| --- | --- | --- | --- | --- | --- | --- |
| Device Properties | Query only | N/A | N/A | N/A | N/A | N/A |
| Parallel Reduction | Shared-mem tree | 128 threads/blk | 6.26 ms | 0.16 ms | 38× | Compute (shared mem) |
| Matrix Addition | Element-wise | 16×16 block | 203.37 ms | 2.52 ms | 81× | Memory bandwidth |

## Final Conclusions

GPU parallelism delivers 38–81× speedups over serial CPU code on real workloads — even on a mid-range laptop GPU (RTX 2050 4GB).

Shared memory is critical for compute-bound kernels (reduction): it removes repeated global memory traffic and allows fast intra-block communication at ~50× lower latency.

Memory-bound kernels (matrix addition) benefit from wide memory buses and thousands of concurrent threads that hide latency, even without shared memory optimization.

Strong scaling analysis reveals diminishing returns beyond the optimal thread count, consistent with Amdahl's Law — serial overheads (CPU partial-sum accumulation, memory transfers) cap speedup.

The toolkit/driver mismatch (CUDA 13.1 vs driver 13.0) taught an important practical lesson: always compile with explicit -gencode arch=compute_XX,code=sm_XX to emit binary SASS directly and avoid runtime PTX rejection.

The optimal block size for 1D kernels is typically 128–256 threads; for 2D kernels, 16×16 is the established best-practice on modern NVIDIA GPUs.